IMAGE SENSOR SIMULATION FOR DASHCAM
Researcher: Lê Quang Nhật Nam

- Overview

- Simulation on Vivado

1/ Counter 8 bits

A 8-bit counter that generates the ramp code for the ADC conversion process.

- SystemVerilog code:

module counter(input logic clk,

input logic reset,

output wire[width-1:0] out);

parameter width = 8;

logic[width-1:0] count = 0;

always_ff @(posedge clk or posedge reset) begin

if (reset) begin

count <= 0;

end else begin

count <= count + 1;

end

end

assign out = count;

endmodule

- Waveform:

When the reset signal is at 0 (not activated), the counter starts to operate on the clk pulse. Each rising edge of clk increases the value of out[7:0] by 1, starting from 00, 01, 02… up to 255 because this is an 8-bit counter. The clock period in this waveform is about 10 ns, so each count value lasts for 10 ns before moving to the next value. This confirms that the counter works correctly: incrementing the count value steadily on the clock pulse when the reset is not activated.

3/ FSM

- SystemVerilog code:

module pixelState(

input logic        clk,

input  logic      reset,

output logic erase,

output logic expose,

output logic[3:0] read,

output logic convert

);

//State duration in clock cycles

parameter integer c_erase = 5;

parameter integer c_expose = 255;

parameter integer c_convert = 255;

parameter integer c_read = 5;

//------------------------------------------------------------

// State Machine

//------------------------------------------------------------

parameter ERASE=0, EXPOSE=1, CONVERT=2, READ=3, READ2=4, IDLE=5;

logic               convert_stop;

logic [2:0]         state,next_state;   //States

integer           counter;            //Delay counter in state machine

// Control the output signals

always_ff @(negedge clk ) begin

case(state)

ERASE: begin

erase <= 1;

read <= 4'b0;

expose <= 0;

convert <= 0;

end

EXPOSE: begin

erase <= 0;

read <= 4'b0;

expose <= 1;

convert <= 0;

end

CONVERT: begin

erase <= 0;

read <= 4'b0;

expose <= 0;

convert = 1;

end

READ: begin

erase <= 0;

read <= 4'b1100;

expose <= 0;

convert <= 0;

end

READ2: begin

erase <= 0;

read <= 4'b0011;

expose <= 0;

convert <= 0;

end

IDLE: begin

erase <= 0;

read <= 4'b0;

expose <= 0;

convert <= 0;

end

endcase // case (state)

end // always @ (state)

// Control the state transitions.

always_ff @(posedge clk or posedge reset) begin

if(reset) begin

state = IDLE;

next_state = ERASE;

counter  = 0;

convert  = 0;

end

else begin

case (state)

ERASE: begin

if(counter == c_erase) begin

next_state <= EXPOSE;

state <= IDLE;

end

end

EXPOSE: begin

if(counter == c_expose) begin

next_state <= CONVERT;

state <= IDLE;

end

end

CONVERT: begin

if(counter == c_convert) begin

next_state <= READ;

state <= IDLE;

end

end

READ:

if(counter == c_read) begin

state <= IDLE;

next_state <= READ2;

end

READ2:

if(counter == c_read) begin

state <= IDLE;

next_state <= ERASE;

end

IDLE:

state <= next_state;

endcase // case (state)

if(state == IDLE)

counter = 0;

else

counter = counter + 1;

end

end // always @ (posedge clk or posedge reset)

endmodule

- Simulation

4/ PIXEL_SENSOR

- SystemVerilog code:

module PIXEL_SENSOR

(

input  logic      VBN1,

input  logic      RAMP,

input  logic      RESET,

input  logic      ERASE,

input  logic      EXPOSE,

input  logic      READ,

inout  [7:0]      DATA

);

// ---- analog-like params ----

real  v_erase = 1.2;

real  lsb     = v_erase/255.0;

parameter real dv_pixel = 0.5;

// ---- pixel state ----

real  tmp;               // ?i?n áp pixel (mô ph?ng)

real  adc;               // ramp analog (mô ph?ng)

logic cmp;               // comparator tripped

logic [7:0] p_data, next_p_data; // latch mã ??m

// ---- ERASE + CONVERT (??ng b?) ----

always_ff @(posedge RAMP or posedge ERASE) begin

if (ERASE) begin

tmp    <= v_erase;

adc    <= 0.0;

cmp    <= 1'b0;

p_data <= 8'h00;

end

else begin

adc <= adc + lsb;

if (adc > tmp) cmp <= 1'b1;

p_data <= next_p_data;    // c?p nh?t latch theo next-state

end

end

// ---- EXPOSE (tích l?y ánh sáng) ----

always_ff @(posedge VBN1) begin

if (EXPOSE)

tmp <= tmp - (dv_pixel*lsb * ({$random}%2));

end

// ---- next-state cho p_data ----

always_comb begin

next_p_data = p_data;       // m?c ??nh gi? nguyên

if (!cmp)                   // tr??c khi comparator trip => bám DATA

next_p_data = DATA;

end

// ---- READOUT ----

// READ=1 -> pixel lái bus b?ng p_data; READ=0 -> nh? bus

assign DATA = READ ? p_data : 8'bZ;

endmodule

- Simulation:

5/ PIXEL_ARRAY 2x2

- SystemVerilog code:

module pixelArray(

input logic      VBN1,

input logic      RAMP,

input logic      RESET,

input logic      ERASE,

input logic      EXPOSE,

input [3:0]      READ,

inout [(N*8-1):0]      DATA

);

parameter real    dv_pixel = 0.5;  //Set the expected

parameter integer N = 2;

genvar i;

genvar j;

generate

for(j=0; j<N; j++) begin

for(i=0; i<N; i++) begin

PIXEL_SENSOR  #(.dv_pixel(dv_pixel))  ps1(VBN1,

RAMP,

RESET,

ERASE,

EXPOSE,

READ[i+(j*N)],

DATA[(1+i)*8-1:i*8]);

end

end

endgenerate

endmodule

- Simulation: